KinematiK  ·  Fluent validation run-sheet

How to reproduce a KinematiK aero point in ANSYS Fluent and feed the result back in. Hand this to whoever drives Fluent.

# What you're doing, and what you're not

KinematiK gives you an aero coefficient for each ride-height point. That number is an in-house analytic surrogate — it does not see your geometry. The job here is to run the same operating point through Fluent on your actual meshed car, then read the Fluent result back into KinematiK so it becomes the trusted aero map.

|  | Set expectations before you start Do not expect KinematiK's surrogate and your Fluent number to match closely — different things. The surrogate is a sign-and-magnitude sanity check. Your Fluent run is the real data. The point of this exercise is to get Fluent's coefficients into KinematiK, not to reconcile the two. |
| --- | --- |

# The loop, at a glance

- In KinematiK, write the Fluent journal for the point(s) you want to validate.

- Open the journal, confirm the three per-run inputs (case file, wall zones, inlet zone).

- Run it in licensed Fluent. First run: do it interactively to lock the syntax to your version.

- Get the coefficient CSV out: <case>_coeffs.csv with header Cl,Cd,Cs,CmPitch,converged.

- Back in KinematiK, the reader folds that number into the tunnel correlation. Done.

# Step 1 — Write the journal from KinematiK

Each ride-height point produces one journal named after its attitude, e.g. case_rp000_pp008_yp000_h0018_v270.jou. Programmatically:

from suspension.aero import get_backend from suspension.aero import windtunnel as wt from suspension.aero.cfd import CaseSpec rh = wt.RideHeights(front_mm=18, rear_mm=40, speed_ms=27, wheelbase_mm=1550) att = wt.ride_heights_to_attitude(rh) spec = CaseSpec(attitude=att, geometry_path='car.msh', reference_area_m2=1.0, reference_length_m=1.55, rho=1.225, extra={'case_file': 'runs/2026-06-21/car_FRH18.cas.h5', 'wall_zones': ['floor','wing_f','wing_r','body'], 'inlet_zone': 'inlet'}) backend = get_backend('fluent') # in-house estimate + runnable .jou backend.write_case(spec, 'out/') # writes out/<case>.jou

The reference area, length, density and velocity in the deck are taken straight from spec — the same values KinematiK normalises its coefficients by. Do not change them in Fluent; that's the whole point of them matching.

# Step 2 — Confirm the three per-run inputs

KinematiK can't know how your case is named or zoned, so three things are parameterised. They sit in a labelled block at the top of the journal. Confirm them before every run that differs:

| Input | What it is / where it comes from |
| --- | --- |
| case_file | The meshed case Fluent reads (.cas / .msh / .cas.h5). Varies by run — set it in spec.extra['case_file']. |
| wall_zones | The boundary name(s) the lift/drag/side reports sum over. One zone ('car') or several ('floor','wing_f',…). Must match your mesh's zone names exactly. |
| inlet_zone | The velocity-inlet boundary name (default 'inlet'). The outlet is assumed 'outlet'. |

|  | If a name is wrong Fluent will error with 'zone not found' or silently report zero force. If your downforce comes out as ~0 or absurd, a misnamed wall zone is the first thing to check — not the physics. |
| --- | --- |

# Step 3 — Run it (lock the syntax on the first one)

|  | The single most important thing on this sheet The TUI command strings in the journal — the velocity-inlet prompts, the force-monitor setup, and especially the report-to-CSV export — are Fluent-version-specific. They are a strong scaffold, not a guaranteed first-run script. On a new Fluent version, run the FIRST case interactively, watch which prompt the deck trips on, fix that one line, and save the corrected journal as your template for the rest of the sweep. |
| --- | --- |

Headless, once you trust the deck:

fluent 3ddp -g -i case_rp000_pp008_yp000_h0018_v270.jou ;; single core fluent 3ddp -g -t8 -i case_rp000_pp008_yp000_h0018_v270.jou ;; 8-core parallel

What the deck does, in order: read case → set reference values → k-omega SST → set the inlet vector (yaw and pitch already folded in) → add a lift-coefficient monitor for convergence → initialise → iterate → export the coefficient CSV → write the solved case.

# Step 4 — Get the coefficient CSV out

KinematiK reads one file per case: <case>_coeffs.csv. It takes the last data row. Exact format:

Cl,Cd,Cs,CmPitch,converged 2.80,1.06,0.01,0.17,1

|  | Sign convention — read this twice Fluent reports lift up-positive, so real downforce shows as a positive Cl here. Write Fluent's native value — do not negate it. KinematiK flips the sign on read (its convention is C_l negative = downforce). If you pre-negate, you double-flip and your downforce becomes lift. |
| --- | --- |

## If the auto-export line doesn't cooperate

Some Fluent builds name the report-to-file command differently, and the clean-CSV export doesn't exist in every version. If the export step fails, fall back to writing the file by hand — run the three coefficient reports interactively, read the values off the console, and write two lines:

echo "Cl,Cd,Cs,CmPitch,converged" > case_..._coeffs.csv echo "2.80,1.06,0.01,0.17,1" >> case_..._coeffs.csv ;; converged = 1 if the Cl monitor settled, else 0

KinematiK reads that hand-written file identically — it only cares about the header and the last row.

# Step 5 — Read it back into KinematiK

Point the reader at the folder holding the CSV:

res = backend.read_fluent_csv(spec, 'out/') print(res.c_lift, res.c_drag, res.converged) ;; Fluent Cl=+2.80 -> res.c_lift = -2.80 (downforce, sign flipped)

From here it flows into the tunnel-vs-CFD correlation in the app's wind-tunnel tab: drop the per-point coefficients into the Step 3 results CSV (front_mm, rear_mm, speed_ms, c_lift, c_drag) and KinematiK lines them up against the tunnel map, point by point.

# Gotchas worth pinning above the desk

- Reference area mismatch is the silent killer. If Fluent's reference area ≠ KinematiK's, every coefficient is off by a constant ratio and looks like a physics disagreement. They're set from spec — leave them.

- Pitching moment needs a moment centre. The deck does not set one (it's geometry-dependent). If you care about aero balance (CmPitch), set the moment reference point in your case before running, or that column is meaningless.

- 'converged' is a real flag. Write 0 if the Cl monitor never settled. KinematiK treats an unconverged point as a flag, not a number — better an honest hole than a fake force.

- Moving floor vs fixed floor. If your tunnel map is moving-belt and your Fluent floor is fixed, you're comparing two different measurements. Match the ground condition or expect underbody downforce to disagree.

- One zone or many — be consistent. If wall_zones lists floor + wings + body, every report sums over all of them. Use the same zone set for every point in a sweep, or the points aren't comparable.

KinematiK is the consumer of Fluent here, not the thing Fluent validates. The trustworthy aero map is the one your solver produces; the surrogate is scaffolding. Get the CSV right and the rest is plumbing KinematiK already handles.